## SECTION 1 — WHAT A ROUGH-IN IS

A plumbing rough-in is the hidden stage of plumbing: pipes, drains, vents, and supply lines installed inside walls, floors, and ceilings before drywall or finishes go up.

This is where the real craftsmanship happens—nobody sees it, but everything depends on it.

If the rough-in is wrong, the finish can never hide it.

## SECTION 2 — OBJECTIVES

- Establish correct supply & drain paths

- Maintain vent integrity

- Respect slope requirements

- Position stub-outs correctly for fixtures

- Pass inspection without callbacks

The rough-in is about future-proofing, not shortcuts.

## SECTION 3 — REQUIRED TOOLS

- PEX expander/crimp tool

- Hole saws

- Levels & laser layout tools

- Pipe cutter

- Torch (for copper installs)

- Stud guard plates

- Pipe supports & clamps

If the installer doesn’t have a level, you don’t have a plumber—you have a gambler.

## SECTION 4 — CORE MATERIALS

- PEX A or B

- PVC or ABS for DWV

- 90° / 45° fittings

- Wyes (never tees on drains)

- Stud shoes & nail plates

- Ball valves for shutoffs

## SECTION 5 — STANDARD WORK SEQUENCE

- Locate main water source and drains

- Snap lines and layout fixture positions

- Run drain lines first (gravity rules everything)

- Add vents and tie into main stack

- Install supply lines

- Cap & pressure test

- Inspection

Order never changes: drain → vent → supply.

## SECTION 6 — COMMON ROUGH-IN FAILURES

- Using sanitary tees where wyes are required

- No vent or shared vent without proper sizing

- Drains sloped incorrectly

- Supply lines too tight—no expansion allowance

- No nail plates → screws pierce pipes later

Everything wrong in a house starts with someone rushing the rough-in.

## SECTION 7 — INSPECTION CHECKPOINTS

- ¼” per foot slope

- Proper vent distance to trap (critical)

- Pressure test hold

- Fixture clearances

- Correct pipe sizing

If the inspector smiles, you did it right.