## SECTION 1 — WHAT THE PANEL DOES

The panel is the electrical command center. It receives power from the utility transformer and distributes it safely to circuits throughout the building.

It is the closest thing a house has to a brain.

## SECTION 2 — BREAKER TYPES

- Standard — basic overload protection

- GFCI — wet areas, saves lives

- AFCI — prevents arc fires, now code in many rooms

- Dual Function — GFCI + AFCI

Most DIY misunderstandings start at the panel.

## SECTION 3 — SERVICE SIZE

Common residential sizes:

- 100A — old houses

- 150A — acceptable

- 200A — modern standard

- 400A — large custom homes

Undersizing is future regret.

## SECTION 4 — PANEL RULES

- Ground and neutral split in subpanels

- Neutrals get one terminal each

- No double-taps unless approved terminals

- Label everything

Panels are not storage shelves.

## SECTION 5 — COMMON FAILURES

- Backfed breaker without hold-down kit

- Mis-sized service conductors

- Incorrect bus stab usage

- No bonding jumper on metal enclosures

If the panel is wrong, everything downstream is wrong.