## SECTION 1 — THE PURPOSE OF FLASHING

Flashing isn’t a sticker — it’s the water traffic control system.

Windows don’t leak because of windows.

They leak because someone didn’t tell water where to go.

## SECTION 2 — THE WATER MANAGEMENT PYRAMID

- Slope the sill

- Install pan flashing

- Side flashing overlaps sill

- Head flashing overlaps sides

- Never reverse-lap anything

Everything overlaps like shingles — gravity has rules.

## SECTION 3 — INSTALLATION STEPS

- Dry-fit window

- Check plumb, level, square

- Apply proper sealant

- Shim sides, never bottom

- Fasten per manufacturer specs

- Verify operation before foaming

- Fill gaps with low-expansion foam only

The foam is not insulation — it’s a gasket.

## SECTION 4 — COMMON FAILURES

- Reverse-lapped flashing → guaranteed rot

- No sill slope → standing water

- Over-foaming → bowed frames

- Fasteners through fins → warranty voided

Every leak has a witness: impatience.