### 1. WHAT THIS IS

Replacing an exterior door slab or the entire pre-hung unit to improve security, weather resistance, curb appeal, and energy efficiency.

### 2. WHEN THIS IS NEEDED

- Air leaks or daylight visible around door

- Soft, rotten jamb or threshold

- Loose hinges or sagging frame

- Difficulty locking or closing

- High energy usage + draft complaints

### 3. REQUIRED SKILLS + TOOLS

Skills

- Alignment + hinge mortising

- Weatherstripping integration

- Understanding swing + handing

Tools

- Drill/driver

- Oscillating tool

- Hammer + chisel

- Level + shims

- Caulk gun

### 4. MATERIALS NEEDED

- Pre-hung exterior door OR replacement slab

- Composite shims

- Exterior-grade caulk

- Weatherstripping

- Threshold seal

### 5. STEP-BY-STEP WORKFLOW

Remove old door + interior trim

Dry-fit door + confirm swing direction

Plumb hinge side + insert shims

Secure jamb screws into framing

Seal threshold + apply caulk

Install hardware + weatherstripping

Test latch/lock engagement

Adjust strike plate if required

### 6. CODE & COMPLIANCE NOTES

- Fire-rated doors required between garage & living space

- Egress rules in some jurisdictions

- Smart locks must meet UL security standards

### 7. SAFETY TRAPS

⚠ Fasteners into drywall, not framing = instant sag

⚠ Wrong threshold seal = wind + water intrusion

⚠ No sill pan in wet climates = rot

⚠ Misaligned hinges create binding over time

### 8. TIME + COST EXPECTATIONS

DIY: 3–6 hrs

Pro install: $900–$4,200 (style/type dependent)

### 9. CONTRACTOR SCAM SIGNALS

❌ “No need for shims”—major red flag

❌ No sill pan or threshold seal

❌ Decorative doors with non-rated glass near garage

❌ Inflated hardware costs

### 10. HIRE VS DIY

Hire if:

- Structural jamb rot

- Door size modification required

DIY if:

- Like-for-like replacement

### 11. RECOMMENDATION BOOSTERS

- Provide hinge tension test video

- Leave lock lubrication + care guide

- Offer storm door or smart lock upgrade