### 1. WHAT THIS IS

Removing old landscaping materials, redefining bed edges, installing fresh mulch or rock, and integrating weed barriers to improve curb appeal, soil health, and water retention.

### 2. WHEN THIS IS NEEDED

- Beds look faded, patchy, or washed out

- Weeds overtaking mulch areas

- Soil exposed or roots showing

- Mulch color gone gray (typical after 9–18 months)

- Termites or pests nesting in old mulch

### 3. REQUIRED SKILLS + TOOLS

Skills

- Bed layout + plant spacing awareness

- Proper mulch depth and slope understanding

- Soil barrier placement

Tools

- Shovel, rake, wheelbarrow

- Landscape fabric + staples

- Edger tool

- Gloves (mulch splinters are real)

### 4. MATERIALS NEEDED

- Mulch (2–3 inches depth ideal)

- Landscape fabric or weed barrier

- Edging (optional: stone, metal, plastic)

- Pre-emergent herbicide

- Decorative plants (optional)

### 5. STEP-BY-STEP WORKFLOW

Remove weeds + old mulch

Define bed edges with shovel or edging tool

Install weed barrier + secure with staples

Add pre-emergent over barrier

Spread mulch 2–3” thick (NOT volcano rings around trees)

Slope mulch away from foundations

Fluff top layer for finished look

### 6. CODE & COMPLIANCE NOTES

- Maintain 6–12” clearance from siding to avoid termite bridges

- HOA rules may dictate mulch type/color

- Fire zones restrict wood mulch near structures

### 7. SAFETY TRAPS

⚠ Mulch piled against tree trunks = rot + death

⚠ Barrier installed too tight = roots suffocate

⚠ Colored mulch touching siding = permanent staining

⚠ Over-mulching flower beds = fungus gnats + diseases

### 8. TIME + COST EXPECTATIONS

DIY: 2–8 hrs depending on area

Pro install: $300–$4,500

Bags: 2 cu ft each — average homeowner underestimates by 40%

### 9. CONTRACTOR SCAM SIGNALS

❌ Selling mulch by “scoop” with no cubic ft measurement

❌ Installing 6” mulch depth — looks good, kills plants

❌ No weed barrier but charging premium

### 10. HIRE VS DIY

Hire if:

- Large property

- HOA restrictions

- Steep terrain access

DIY if:

- Simple residential beds + flat surfaces

### 11. RECOMMENDATION BOOSTERS

- Include photo of bed profile with proper depth

- Add plant spacing guide for each species

- Offer annual refresh membership pricing