### 1. WHAT THIS IS

Removing deteriorated mortar from masonry joints and replacing it with fresh mortar to restore structural stability, prevent water intrusion, and preserve brick or stone surfaces.

### 2. WHEN THIS IS NEEDED

- Mortar crumbles when rubbed

- Gaps forming between brick joints

- Water stains along walls

- Loose brick faces

- Home older than 40+ years

### 3. REQUIRED SKILLS + TOOLS

Skills

- Mortar color matching

- Joint clearing + repack

- Protecting brick faces from damage

Tools

- Angle grinder + mortar blade

- Tuckpointing trowel

- Joint raker

- Hammer + chisel

- Dust mask + goggles

### 4. MATERIALS NEEDED

- Mortar mix compatible with brick age (lime vs Portland)

- Joint brushing tools

- Sponge + misting spray

- Sealer (optional)

### 5. STEP-BY-STEP WORKFLOW

Remove loose mortar to proper depth

Brush joints clean

Mix mortar to peanut butter consistency

Pack joints securely with trowel

Tool finish to original profile

Mist lightly to prevent rapid dry-out

Cure as directed

Seal if required

### 6. CODE & COMPLIANCE NOTES

- Historic brick requires lime mortar—Portland cement damages old masonry

- Chimneys may require permit

- Freeze-thaw regions need breathable sealers

### 7. SAFETY TRAPS

⚠ Grinding too deep = brick face spalling

⚠ Using wrong mortar type = catastrophic failure

⚠ Over-wetting = weakened mortar

⚠ Sealing non-breathable surfaces traps moisture

### 8. TIME + COST EXPECTATIONS

DIY: 3–10 hrs depending on area

Pro: $600–$8,500 depending on elevation

Chimneys add complexity and cost

### 9. CONTRACTOR SCAM SIGNALS

❌ Using gray mortar with red brick mismatch

❌ Incorrect mortar type for era

❌ No curing misting schedule

❌ Superficial smear instead of proper depth repacking

### 10. HIRE VS DIY

Hire if:

- Historic brick involved

- Chimney work

- Tall walls needing scaffolding

DIY if:

- Small ground-level areas

### 11. RECOMMENDATION BOOSTERS

- Leave mortar color match sample

- Provide moisture ingress report

- Add optional chimney cap upgrade offer