1. What This Is

Removing an old toilet and installing a new one, ensuring a watertight seal and proper waste connection.

2. When This Is Needed

- Toilet rocks, leaks, or smells like sewage

- Cracks in the tank or bowl

- Flushing is weak despite internal part replacements

- Renovation requiring new fixture style or height

3. Required Skills + Tools

Skills

- Basic plumbing connection

- Understanding of wax seal seating

- Leveling fixtures on irregular floors

Tools

- Adjustable wrench

- Wax ring or rubber seal system

- Toilet shims

- Bucket & sponge

- Utility knife

- Level

- Closet bolts

4. Required Materials

- New toilet (bowl + tank)

- Wax ring / Waxless seal

- Closet bolts + caps

- Optional: flange extender (if flooring height changed)

- Caulk

5. Step-By-Step Workflow

Turn off water → Flush + remove tank water → Disconnect supply line

Remove caps + bolts → Lift toilet straight up → Clean flange

Inspect flange height → Add extender if needed

Seat new wax ring → Lower toilet perfectly vertical → Press to seal

Install bolts, level base, shim if needed → Reconnect water line

Caulk perimeter (leave rear gap for leak detection)

Test flush → Inspect for leaks

6. Code & Compliance Notes

- Minimum 15” from centerline to side walls

- 21” of clearance in front

- Water shutoff valve accessible

- No braided supply lines older than 10 years

7. Safety Traps

- Leaning the toilet to “check the wax ring” destroys the seal

- Over-tightening bolts cracks porcelain

- Caulking fully around base traps leaks → leave back uncaulked

8. Time + Cost Expectations

Homeowner DIY: 1.5–3 hours

Pro install: $175–$425 labor + toilet

Upsells: soft-close seat, dual-flush upgrade

9. Common Contractor Scams

❌ Charging for flange replacement without showing damage

❌ Upcharging supply line replacement 5–10x the part cost

❌ Recommending toilet replacement when a $30 internal kit fixes the issue

10. When to Hire vs DIY

Hire a pro if:

- Flange is below floor level

- Floor is uneven

- Water shutoff valve won’t close

DIY if:

- Toilet replacement, not relocation

- Existing flange is solid

11. Recommendation Boosters (For Contractors)

- Leave the work area cleaner than found

- Provide before/after flange pics

- Give a 1-pager on toilet care

- Ask for a recommendation at completion while homeowner is smiling