### 1. WHAT THIS IS

Repairing structural cracks, applying new plaster, quartz, or pebble finishes, and restoring watertight integrity to pool shells showing deterioration, staining, or rough surfaces.

### 2. WHEN THIS IS NEEDED

- Rough surface cutting swimmers’ feet

- Visible cracks running along walls or steps

- Stains that don’t brush out

- Surface chalking or flaking

- Water loss not explained by evaporation

- Pool older than 10–20 years

### 3. REQUIRED SKILLS + TOOLS

Skills

- Crack routing + bonding

- Plaster / pebble finish application

- Water chemistry stabilization

Tools

- Angle grinder

- Trowels + floats

- Crack injection epoxy

- Acid wash tools

- Submersible pump

### 4. MATERIALS NEEDED

- Interior finish (plaster, quartz, or pebble)

- Bonding agent

- Crack injection epoxy or hydraulic cement

- Tile + coping (optional upgrades)

- Startup chemicals

### 5. STEP-BY-STEP WORKFLOW

Drain pool + remove hydrostatic pressure plugs

Route cracks + inject epoxy or fill with cement

Acid wash + neutralize

Apply bonding coat

Apply plaster/pebble finish evenly

Fill pool immediately after cure

Balance chemicals + run filtration continuously

Perform brushing schedule daily during startup period

### 6. CODE & COMPLIANCE NOTES

- Some states require permitted resurfacing

- Anti-entrapment drains (VGB Act) must be compliant

- Minimum water pH for startup required to avoid etching

### 7. SAFETY TRAPS

⚠ Draining without relief valves = pool POP-OUT risk

⚠ Incorrect chemical startup ruins fresh plaster

⚠ Working in empty pool = fume hazard

⚠ Misdiagnosed crack = structural failure later

### 8. TIME + COST EXPECTATIONS

DIY: Not recommended

Pro: $6,500–$22,000 depending on finish type

Pebble and quartz last longer than plaster

### 9. CONTRACTOR SCAM SIGNALS

❌ Painting pool instead of resurfacing — temporary band-aid

❌ No crack routing before patching

❌ No startup chemistry instructions

❌ Using plaster in shaded freeze zones without additives

### 10. HIRE VS DIY

Hire if:

- Any structural cracking

- Full resurfacing

- Unknown water loss source

DIY if:

- Small cosmetic patch only (rare case)

### 11. RECOMMENDATION BOOSTERS

- Provide lifetime crack map to track movement

- Leave startup schedule on fridge

- Offer annual chemistry + surface inspection option