### 1. WHAT THIS IS

Removing and replacing an entire siding system—wood, vinyl, metal, fiber cement, etc.—to improve structural integrity, energy efficiency, curb appeal, and weather resistance.

### 2. WHEN THIS IS NEEDED

- Siding aged, brittle, warped, or cracked

- Rot present behind boards

- Repeated paint failures

- Major hail or storm damage

- Home receiving insulation upgrade or envelope retrofit

### 3. REQUIRED SKILLS + TOOLS

Skills

- Full wall tear-off and substrate assessment

- Flashing & moisture barrier installation

- Correct fastener placement and siding patterning

Tools

- Siding nailer or coil gun

- Circular saw with appropriate blade

- Level + chalk line

- Snips (vinyl/metal)

- Scaffold or safe ladder system

### 4. MATERIALS NEEDED

- New siding system (type + trim pieces)

- House wrap or weather barrier

- Flashing tape + corner boards

- Fasteners specific to siding

- Paint/finish (wood/fiber cement)

### 5. STEP-BY-STEP WORKFLOW

Remove old siding + trim

Inspect sheathing + framing

Install new weather barrier with taped seams

Flash openings and penetrations

Install starter strips

Run siding bottom-up with correct overlaps

Install trim, corners, and accessories

Caulk strategic joints (NOT weep paths)

Finish paint or stain if required

Clean site + dispose of debris

### 6. CODE & COMPLIANCE NOTES

- Weather barrier mandatory in nearly all jurisdictions

- Fastener penetration must meet manufacturer spec

- Maintain expansion gaps per product manual

- Fire ratings apply near lot lines in some cities

### 7. SAFETY TRAPS

⚠ Vinyl nailed tight → cracks in winter

⚠ Covering rotten sheathing → structural failure later

⚠ Fiber cement cutting without respirator → silica inhalation risk

⚠ No starter strip → bugs + water intrusion

### 8. TIME + COST EXPECTATIONS

DIY: 3–10 days (not recommended for most homeowners)

Pro install: $9,500–$58,000 depending on size and product

Materials: Vinyl (low), Fiber Cement (mid), Cedar (high)

### 9. CONTRACTOR SCAM SIGNALS

❌ No weather barrier → catastrophic future rot

❌ Wrong fasteners → rust bleed-through

❌ “Close enough” alignment on corners

❌ Reusing trim pieces that are compromised

### 10. HIRE VS DIY

Hire if:

- Whole house project

- Fiber cement or metal siding used

- Two-story home and/or many penetrations

DIY if:

- Small shed or outbuilding only

### 11. RECOMMENDATION BOOSTERS

- Provide full photo log of wall prep + house wrap

- Leave touch-up paint and product manuals

- Verify attic ventilation openings remain clear

- Offer a spring inspection package