#### 1. WHAT THIS TOOL IS

An oscillating multi-tool is a precision cutting and shaping tool that vibrates a blade side-to-side at high speed, allowing controlled cuts in tight spaces, flush trims, undercuts, and detailed adjustments that no other saw can safely perform.

It is the problem-solver of the flooring trade — the tool you reach for when nothing else works.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use an oscillating multi-tool when:

- Undercutting door trim to slide flooring underneath

- Cutting flooring planks around vents, pipes, or corners

- Trimming baseboards, transitions, and thresholds

- Removing damaged plank sections without dismantling the floor

- Cutting materials flush against walls or obstacles

If a flooring job gets stuck, the multi-tool unsticks it.

Every installer eventually learns:

The job always needs this tool — eventually.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

✔ Match the blade to the material (wood, metal, PVC)

✔ Start cuts slowly to avoid blade wander

✔ Use the tool flat and supported for flush accuracy

✔ Keep RPMs high for clean cuts, low for control

✔ Work dust-free with adapters when indoors

Amateurs:

❌ Melt flooring by using wrong blade speeds

❌ Use wood blades on metal nails or screws

❌ Leave jagged cuts hidden under trim

❌ Force the tool instead of letting the oscillation work

TradeScout Commandment:

“A multi-tool is not a grinder. Patience is the blade.”

#### 4. BLADE TYPES THAT MATTER

Wood blade — trim and undercuts

Bi-metal blade — nails, screws, metal thresholds

Scraper blade — remove old adhesive or caulk

Grout blade — tile removal and repair

A pro doesn’t try blades — a pro selects blades.

#### 5. SAFETY TRAPS

⚠ Blades get hot enough to burn skin

⚠ Running tool unsupported = kickback micro-jumps

⚠ Metal cuts throw sparks near flooring adhesives

⚠ Cutting blindly can strike wiring or pipes

A multi-tool is safe only as long as the operator is smarter than the blade.

#### 6. MAINTENANCE CHECKLIST

- Replace dull blades — a dull blade burns, not cuts

- Keep vents clear of dust buildup

- Periodically tighten blade clamp

- Inspect power cord or battery lock

- Store blades sorted by material

A pro protects the tool that protects the timeline.

#### 7. CUSTOMER TRUST SIGNALS

Homeowners see mastery when contractors:

📌 Undercut trim instead of notching planks

📌 Make perfectly fitted shapes around door jambs

📌 Avoid filler strips or sloppy transitions

📌 Perform repairs without ripping up entire rooms

This tool silently communicates:

“I solve problems without excuses.”

That is the TradeScout advantage — craftsmanship that earns recommendations without asking.