#### 1. WHAT THIS TOOL IS

Refrigerant manifold gauges are diagnostic pressure instruments used to measure the high-side and low-side pressures inside HVAC refrigeration circuits. They reveal system performance, refrigerant charge status, and compressor health.

If the HVAC system has a heartbeat, manifold gauges are the stethoscope.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use manifold gauges when:

- Charging or recovering refrigerant

- Diagnosing system cooling/heating problems

- Checking for leaks or restrictions

- Verifying compressor suction and discharge pressures

- Evaluating expansion valves and metering devices

Without gauges, HVAC “repairs” are guesses.

With gauges, they’re math.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

✔ Purge air from hoses before connecting

✔ Stabilize pressures before interpreting readings

✔ Cross-reference readings with temperature charts

✔ Use micron gauge during evacuation (not manifold gauge alone)

✔ Document readings for homeowners — data is trust

Amateurs:

❌ Vent refrigerants to the atmosphere — illegal and reckless

❌ Diagnose systems without knowing pressures

❌ Use incorrect refrigerant charts

❌ Overcharge systems hoping “cold equals fixed”

TradeScout Law:

“If you can’t read pressures, you can’t fix comfort.”

#### 4. COLOR CODE REALITY

- Blue hose = Low side / Suction

- Red hose = High side / Discharge

- Yellow hose = Service / Recovery / Charging

Pros don’t guess hoses. They respect them.

#### 5. SAFETY TRAPS

⚠ Incorrect connection = hose whip or refrigerant burns

⚠ Venting refrigerant = EPA violation + $25,000 fines

⚠ Over-pressurization = line rupture and injury

⚠ Using wrong gauge set for refrigerant type = false readings

HVAC is not “dangerous.”

Ignorance in HVAC is dangerous.

#### 6. MAINTENANCE CHECKLIST

- Check rubber seals monthly

- Keep sight glass clean for accurate observation

- Replace hoses annually or when cracked

- Store away from UV exposure

- Verify gauge calibration annually

- Never coil hoses tightly — causes internal damage

A dirty gauge produces dirty data.

#### 7. CUSTOMER TRUST SIGNALS

Pros who earn recommendations:

📌 Explain readings in plain terms

📌 Compare expected vs actual pressures

📌 Photograph gauge readings for records

📌 Pair gauge work with temperature delta measurements

Homeowners don’t understand “specs.”

They understand proof.

Data = trust.

Trust = referrals.

Referrals = the TradeScout economy.