## SECTION 1 — WHAT CONCRETE REALLY IS

Concrete is not “cement.”

Cement is the ingredient.

Concrete is the mixture — aggregate, sand, cement, and water — combined into a structural mass that becomes stronger over time.

Concrete doesn’t harden.

It cures — and curing is chemistry with consequences.

## SECTION 2 — WHERE CONCRETE IS USED

- Foundations & slabs

- Driveways & flatwork

- Footings & stem walls

- Sidewalks, porches, patios

- Retaining structures

- Columns & beams

Concrete is the skeleton. Everything else is decoration.

## SECTION 3 — MIX BASICS

Concrete strength is measured in PSI:

- 2,500 PSI — sidewalks / flatwork

- 3,000–3,500 PSI — residential slabs

- 4,000+ PSI — heavy loads / garages / commercial

Add water for convenience, lose strength forever.

If the mix truck driver offers “just a splash,” he’s offering future cracks.

## SECTION 4 — REINFORCEMENT

Concrete is strong in compression, weak in tension.

That’s why rebar, mesh, and fibers exist.

Reinforcement doesn’t add strength — it prevents failure.

A slab without reinforcement is a promise waiting for a season change.

## SECTION 5 — ESSENTIAL TOOLS

- Screed boards & floats

- Levels & laser leveling tools

- Rebar cutters/benders

- Bull floats / magnesium floats

- Forms & stakes

- Vibrators for consolidation

Concrete doesn’t care how confident you are — it cares how prepared you are.

## SECTION 6 — STANDARD WORKFLOW

- Soil preparation and compaction

- Formwork layout

- Reinforcement installation

- Pour and consolidation

- Screeding and finishing

- Curing and protection

Each step is a moment to decide whether the slab lasts decades or months.

## SECTION 7 — COMMON FAILURES

- Overwatering → weak mix

- No control joints → random cracking

- Poor soil prep → settlement

- No vapor barrier → moisture problems

- Incorrect cure time → surface dusting

Concrete doesn’t crack because time passed.

It cracks because someone took a shortcut.

## SECTION 8 — SAFETY CONSIDERATIONS

- Skin exposure to wet concrete → burns

- Rebar trip hazards

- Heavy lifting

- Silica dust when cutting

Concrete removes egos and fingerprints with equal indifference.

## SECTION 9 — DIY VS PRO

DIY okay:

- Small pads

- Walkways

- Fence post footings

Hire a pro:

- House slabs

- Garage floors

- Structural pours

- Retaining systems

Concrete is cheap once. Expensive twice.

## SECTION 10 — WHAT MAKES A GOOD CONCRETE CREW

They:

- Respect soil before concrete

- Measure twice, pour once

- Place control joints where physics expects them

- Understand curing like religion

- Keep finishers who don’t rush shine over strength

- Leave slabs that stay quiet for decades

Concrete isn’t judged on day one.

It’s judged by what it doesn’t do year ten.