### 1. What This Is

Replacing or installing a ceiling-mounted light fixture or ceiling fan, including safe support, proper electrical connections, and balancing fan blades for noise-free operation.

### 2. When This Is Needed

• Existing fixture flickers, buzzes, or shorts

• Upgrading room airflow or lighting quality

• Fixture produces heat, burnt-plastic smell, or hums

• No ceiling fixture exists & power must be extended

• Homeowner wants reduced energy use (LED + fan combo)

### 3. Required Skills + Tools

Skills

• Basic electrical wiring (hot/neutral/ground ID)

• Reading breaker panel labels

• Using stud finder & working overhead safely

• Fan blade alignment and motor assembly

Tools

• Voltage tester

• Wire strippers + screwdrivers

• Ladder + stud finder

• Electrical box screws & supports

• Torque screwdriver (fan bracket mounting)

• Wire nuts + electrical tape

### 4. Materials Needed

• New fixture or ceiling fan kit

• UL-listed fan-rated electrical box (fan installs only)

• Mounting bracket + hardware

• Wire connectors + pigtails

• LED bulbs (if applicable)

• Optional remote control module

### 5. Step-By-Step Workflow

Turn off breaker → Confirm power is dead

Remove old fixture + inspect box rating

Install fan-rated box if not present

Secure mounting bracket to framing

Assemble motor housing per instructions

Connect wiring: Black = hot, White = neutral, Green/Bare = ground

Hang motor + attach blades (ensure equal screw torque)

Install light kit and bulbs

Restore power → Test speeds + wobble

Fine-tune blade alignment if noisy

### 6. Code & Compliance Notes

• Electrical box MUST be fan-rated if a fan is installed

• All connections made in approved junction box — no open splices

• Ground wire required on all metallic fixtures

• Breaker must match circuit amperage (typically 15A or 20A)

• Bedrooms may require AFCI protection depending on jurisdiction

### 7. Safety Traps

⚠️ Installing a fan on a standard light-rated box → structural failure risk

⚠️ Mixing neutrals from separate circuits → hidden overload

⚠️ Using incorrect screws on mounting bracket → wobble vibration + fire hazard

⚠️ Not tightening blade screws evenly → long-term motor damage

### 8. Time + Cost Expectations

DIY install: 1.5–3 hrs

Pro install: $175–$600

Fan-rated box retrofit: +$80–$250

### 9. Contractor Scam Signals

❌ Charging for “fan wiring upgrade” when existing neutral/hot are adequate

❌ Using drywall anchors instead of structural support

❌ Refusing to show the box rating stamp — always visible

❌ Requiring an unnecessary dedicated circuit

### 10. Hire vs DIY

Hire a Pro If:

• No existing ceiling junction box

• Circuit lacks ground conductor

• Fan is oversized for room or requires bracing

• Multiple switches / 3-way control desired

DIY If:

• Like-for-like fixture replacement

• Box already fan-rated

• You’re comfortable identifying conductors

### 11. Recommendation Boosters (for contractors)

• Leave a blade balancing card + future alignment instructions

• Provide fixture + fan model/serial numbers for warranty

• Offer seasonal fan direction guide (summer vs winter airflow)

• Take before/after shots — huge momentum on rankings