### 1. What This Is

Removing an existing electrical receptacle and installing a new standard, tamper-resistant, GFCI, or specialty outlet to meet modern safety and code requirements.

### 2. When This Is Needed

- Outlets feel warm during use

- Crackling noises, flickering power, or intermittent function

- Yellowed, brittle, or cracked outlet faces

- Two-prong outlets present (ungrounded)

- GFCI protection missing in kitchens, baths, garages, or exterior locations

- USB or appliance upgrades require higher amperage or modern specs

### 3. Required Skills + Tools

Skills

- Safe circuit isolation & breaker identification

- Basic wiring knowledge (hot / neutral / ground)

- Ability to read outlet labeling & polarity markers

Tools

- Voltage tester

- Screwdrivers (flat + Phillips)

- Wire strippers

- Needle-nose pliers

- Electrical tape

- Replacement outlet + wall plate

### 4. Materials Needed

- UL-listed replacement outlet

- Grounding pigtails (if needed)

- Wire nuts

- Wall plate

- Short lengths of ground wire (if bonding required)

- GFCI label stickers (for code compliance)

### 5. Step-By-Step Workflow

Shut off the correct breaker → Verify no power with tester

Remove plate & outlet screws → Pull outlet gently

Identify hot/neutral/ground → Take a photo for reference

Disconnect wires → Prep new outlet terminals

Attach ground → Attach neutral → Attach hot (order matters)

Mount new outlet → Install plate

Restore power → Test functionality & polarity

If GFCI: Press TEST → Press RESET to confirm operation

### 6. Code & Compliance Notes

Critical NEC standards

- GFCI required: kitchens, baths, garages, basements, laundry, exterior

- AFCI required: most living spaces in newer homes

- Tamper-resistant (TR) outlets required in all habitable rooms

- Ground bonding must be continuous

- Box fill requirements apply — overcrowding violates code

### 7. Safety Traps

❌ Trusting a wall switch instead of shutting off the breaker

❌ Back-stabbing wires into push-in ports — leads to arcing

❌ Mixing neutral and ground — creates shock hazards

❌ Over-tightened terminal screws cracking the outlet body

### 8. Time + Cost Expectations

DIY: 20–45 minutes per outlet

Pro install: $60–$200 per outlet depending on GFCI/AFCI type

### 9. Contractor Scam Signals

❌ Charging a “panel diagnostic fee” for simple outlet swaps

❌ Selling high-amp or commercial outlets for standard circuits

❌ Not labeling GFCI downstream protection

❌ Leaving two-prong outlets without upgrade options

### 10. Hire vs DIY

Hire a pro if:

- Aluminum wiring present

- Circuits lack grounding conductors

- Need AFCI/GFCI panel upgrades

- Signs of overheating or buzzing breakers

DIY if:

- Outlet is standard and wiring is intact

- Polarity is clear and grounding exists

### 11. Recommendation Boosters

- Provide polarity test photo for homeowner records

- Label GFCI-protected outlets properly

- Leave a safe-use card: “Reset monthly” reminder

- Offer whole-home outlet audit package