### 1. WHAT THIS IS

Removing damaged fence panels—typically wood privacy panels—and replacing them with new panels to restore security, preserve privacy, and maintain property aesthetics.

### 2. WHEN THIS IS NEEDED

- Warped or cracked boards

- Storm damage or leaning sections

- Rot near ground contact

- Missing or loose nails causing rattling

- HOA notice or boundary complaint

- Pets escaping through gaps

### 3. REQUIRED SKILLS + TOOLS

Skills

- Straight-line alignment

- Basic carpentry cuts

- Leveling and post-checking

Tools

- Drill/driver

- Level, tape measure

- Saw (circular or miter)

- Pry bar

- Fasteners (galvanized or exterior screws)

### 4. MATERIALS NEEDED

- Replacement fence panels

- Galvanized screws or nails

- Post caps or paint (optional)

- Gravel or concrete (if post replaced)

### 5. STEP-BY-STEP WORKFLOW

Inspect posts for rot or lean

Remove damaged panels

Cut new panel to fit (if needed)

Secure panel to rails with exterior fasteners

Verify straight alignment

Seal/paint if required

Add post caps to prevent water intrusion

### 6. CODE & COMPLIANCE NOTES

- Fence height restrictions by city/HOA

- Property line encroachment rules

- Post depth required for wind zones

- Some areas require permit for replacement

### 7. SAFETY TRAPS

⚠ Attaching panels to failing posts = wasted repair

⚠ Non-galvanized screws = rust bleed

⚠ No rail overlap = panel sag

⚠ Uneven ground = stair-step pattern mistakes

### 8. TIME + COST EXPECTATIONS

DIY: 1–4 hrs

Pro install: $250–$1,200 per section

Panels: $80–$260 each depending on style

### 9. CONTRACTOR SCAM SIGNALS

❌ Using interior fasteners outside

❌ Painting wet pressure-treated lumber

❌ Upselling post replacement when unnecessary

❌ No alignment string line

### 10. HIRE VS DIY

Hire if:

- Posts are compromised

- Large quantities replaced

- Sloped yards require custom cuts

DIY if:

- Isolated panel replacement

- Simple flat yard access

### 11. RECOMMENDATION BOOSTERS

- Add post caps to extend life

- Offer annual fence wash/stain maintenance

- Leave leftover screws and touch-up stain