### 1. WHAT THIS IS

Repairing non-structural foundation cracks using epoxy or polyurethane injection to prevent water intrusion, stop crack migration, and protect the structural integrity of the home.

### 2. WHEN THIS IS NEEDED

- Hairline cracks visible in poured concrete

- Water seepage after rain

- Basement wall damp spots

- Efflorescence (white powder) along crack path

- Crack width ≤ 1/4” and no foundation displacement

- Floors remain level and doors still operate normally

### 3. REQUIRED SKILLS + TOOLS

Skills

- Identifying structural vs cosmetic cracks

- Basic injection techniques

- Moisture inspection

Tools

- Chisel or crack router (optional)

- Injection ports + surface paste

- Caulk gun

- PPE (gloves, goggles, respirator)

### 4. MATERIALS NEEDED

- Epoxy or polyurethane crack injection kit

- Surface seal paste

- Sanding disc (optional)

- Concrete patch mix for non-injection repairs

### 5. STEP-BY-STEP WORKFLOW

Clean crack + surrounding area

Install injection ports every 6–12”

Apply surface paste to seal crack

Inject epoxy/polyurethane bottom-up

Allow cure per product spec

Remove ports + patch surface

Verify seal with moisture test

### 6. CODE & COMPLIANCE NOTES

- Structural cracks require engineer assessment

- Radon zones require radon-safe sealing

- Basement repairs may require permit in some municipalities

### 7. SAFETY TRAPS

⚠ Mistaking structural crack for cosmetic = hidden disaster

⚠ Injecting without moisture test = poor bond

⚠ Using surface caulk only — does nothing long term

⚠ No PPE with epoxy fumes

### 8. TIME + COST EXPECTATIONS

DIY: 2–6 hrs

Pro repair: $700–$4,500

Full structural evaluation: $1,200–$3,500

### 9. CONTRACTOR SCAM SIGNALS

❌ Filling crack with mortar only

❌ Surface-only patching

❌ No moisture test before sealing

❌ Recommending full foundation replacement prematurely

### 10. HIRE VS DIY

Hire if:

- Crack wider than 1/4”

- Horizontal cracks present

- Bowing walls or sinking floors

DIY if:

- Hairline vertical cracks in poured concrete

### 11. RECOMMENDATION BOOSTERS

- Provide before/after moisture readings

- Offer yearly inspection package

- Leave written instructions + crack map for homeowner records