### 1. WHAT THIS IS

Removing an aging or failing pool pump and/or filtration unit and installing modern, energy-efficient equipment to improve circulation, reduce electricity usage, and maintain safer water chemistry.

### 2. WHEN THIS IS NEEDED

- Pool water turns cloudy even after treatment

- Pump loudness increases or motor overheats

- Visible leaks around seals or unions

- Pressure gauge stuck high or low

- System over 7–12 years old

- Energy bill spikes during swim season

### 3. REQUIRED SKILLS + TOOLS

Skills

- Basic plumbing alignment + PVC cutting

- Electrical disconnect/reconnect

- Understanding pump flow rates and filter sizing

Tools

- PVC cutter or saw

- Primer + solvent cement

- Pipe wrench

- Screwdrivers + nut drivers

- Multimeter (optional for electrical checks)

### 4. MATERIALS NEEDED

- Replacement pump (variable-speed recommended)

- Sand, cartridge, or DE filter (match capacity to pump)

- Unions, fittings, nipples, check valve (as needed)

- Teflon tape + PVC glue

- O-rings + lubricant

### 5. STEP-BY-STEP WORKFLOW

Shut off power at breaker

Relieve filter pressure

Disconnect plumbing + unions

Remove pump + filter assemblies

Dry-fit new equipment to ensure alignment

Glue PVC connections + install unions

Reconnect electrical lines to pump

Fill filter + prime pump

Run system + verify PSI and flow

Check for leaks and adjust run schedule

### 6. CODE & COMPLIANCE NOTES

- Variable-speed pumps required by law in many states

- Bonding/grounding required on metallic pumps

- DE filters require proper waste disposal system

- Follow manufacturer flow ratings to prevent damage

### 7. SAFETY TRAPS

⚠ Running pump dry burns out seals + motor

⚠ Over-sizing pump overwhelms filter → green pool

⚠ Mixing DE improperly can clog pipes

⚠ Installing unions backwards = leaks forever

### 8. TIME + COST EXPECTATIONS

DIY: 3–6 hrs

Pro install: $1,200–$4,800 depending on pump/filter type

Variable-speed pumps save 50–80% energy cost

### 9. CONTRACTOR SCAM SIGNALS

❌ Selling “bigger pump is better” — FALSE

❌ No PSI reading before/after install

❌ Not replacing unions → forces re-cut later

❌ Removing safety vacuum release system (illegal some areas)

### 10. HIRE VS DIY

Hire if:

- Pump relocation or electrical modifications

- DE systems requiring backwash tie-ins

DIY if:

- One-for-one pump or filter replacement

### 11. RECOMMENDATION BOOSTERS

- Leave PSI log card taped to filter

- Set digital pump schedule for customer

- Provide energy savings breakdown

- Add free first-month check-in visit