### 1. WHAT THIS IS

Repairing damaged, loose, or rotted exterior siding sections to restore weather protection, prevent moisture intrusion, and extend the life of the home’s exterior.

### 2. WHEN THIS IS NEEDED

- Visible cracks, holes, or chips

- Paint peeling or bubbling on siding surface

- Water staining or swelling at lower edges

- Loose boards rattling in wind

- Pest entry points or woodpecker damage

- Isolated rot—not entire wall failure

### 3. REQUIRED SKILLS + TOOLS

Skills

- Recognizing structural vs cosmetic damage

- Removing and reinstalling siding pieces

- Matching materials and finishes

Tools

- Pry bar + siding removal tool (zip tool for vinyl)

- Circular or miter saw

- Caulk gun

- Exterior screws/nails

- Ladder + tape measure

### 4. MATERIALS NEEDED

- Replacement siding boards/panels

- Color-matched paint (fiber cement/wood)

- Caulk rated for exterior joints

- Flashing tape for behind repairs

- Fasteners appropriate for siding type

### 5. STEP-BY-STEP WORKFLOW

Identify affected sections

Remove damaged panels or boards

Assess sheathing for rot → repair if needed

Cut replacement pieces to match profile

Install siding per original pattern

Secure with approved fasteners

Seal joints + penetrations

Paint or finish repaired section

Inspect alignment from street view

### 6. CODE & COMPLIANCE NOTES

- Maintain required clearances from grade (typically 6”)

- Fiber cement requires stainless or hot-dipped galvanized fasteners

- Vinyl siding must float—never nailed tight

- Horizontal joints must shed water

### 7. SAFETY TRAPS

⚠ Matching profile incorrectly = visible forever

⚠ Caulking weep holes = water trapped → rot

⚠ Driving nails too tight on vinyl = cracking in temperature swings

⚠ Ignoring sheathing moisture → mold and insurance denial

### 8. TIME + COST EXPECTATIONS

DIY: 2–6 hrs depending on damage size

Pro repair: $350–$2,500

Materials: $50–$600 depending on siding type

### 9. CONTRACTOR SCAM SIGNALS

❌ Painting raw boards without primer

❌ Using interior-grade caulk outdoors

❌ Ignoring flashing layer

❌ Mismatched patterns passed off as “close enough”

### 10. HIRE VS DIY

Hire if:

- Rot extends beyond siding

- Repairs involve second-story access

- Fiber cement cutting required (silica dust concerns)

DIY if:

- Isolated vinyl/wood damage

- Easy ladder access

### 11. RECOMMENDATION BOOSTERS

- Provide leftover touch-up paint

- Photograph sheathing before re-siding

- Add pest-resistant backing where woodpeckers targeted

- Leave a maintenance schedule