### 1. WHAT THIS IS

Removing an old window unit and installing a modern replacement window into the existing frame—improving insulation, reducing drafts, and increasing home value without full frame reconstruction.

### 2. WHEN THIS IS NEEDED

- Drafts or visible light around window edges

- Fog or moisture between panes (seal failure)

- Windows painted shut or difficult to open

- Energy bills rising year over year

- Rotten sill, warped frame, or cracked glass

### 3. REQUIRED SKILLS + TOOLS

Skills

- Measuring rough openings correctly

- Basic trim carpentry

- Flashing + waterproofing awareness

Tools

- Pry bar + putty knife

- Caulk gun

- Drill/driver

- Level + shims

- Utility knife

### 4. MATERIALS NEEDED

- Retrofit window

- Foam or fiberglass insulation

- Exterior flashing tape

- Exterior-grade caulk

- Trim nails + shims

### 5. STEP-BY-STEP WORKFLOW

Measure existing opening

Remove interior stops + old sash

Dry-fit new window + install shims

Secure unit through side jambs

Insulate perimeter gap

Apply flashing tape + exterior caulk

Reinstall trim + test operation

Verify lock engagement + seal integrity

### 6. CODE & COMPLIANCE NOTES

- Egress requirements apply to bedrooms

- Tempered glass required near doors + wet areas

- U-factor & SHGC vary by climate zone

### 7. SAFETY TRAPS

⚠ Over-insulating jams = window won’t operate

⚠ Caulking without flashing = water intrusion

⚠ Wrong glass type near tubs = code violation

⚠ Reusing rotten sill = failure within a year

### 8. TIME + COST EXPECTATIONS

DIY: 2–4 hrs per window

Pro install: $350–$1,200 per window

High-performance glass increases ROI

### 9. CONTRACTOR SCAM SIGNALS

❌ “Free lifetime caulk”—no such thing

❌ No mention of flashing tape

❌ No energy rating documentation

❌ Pressure sales tactics for unnecessary upgrades

### 10. HIRE VS DIY

Hire if:

- Structural sill damage

- Full-frame replacement required

DIY if:

- Basic retrofit, square frame, minimal rot

### 11. RECOMMENDATION BOOSTERS

- Provide U-factor + SHGC specs

- Leave homeowner with maintenance tips

- Document before/after drafts using smoke test

- Offer screen replacement + annual checkup