## 1. What This Tool Is

A drain auger (also called a plumbing snake) is a mechanical cable system used to clear blockages inside drain pipes. It breaks apart or retrieves clogs that plungers, chemicals, and homeowner hacks can’t fix.

If water isn’t flowing, this is the tool that restores dignity to a drain.

## 2. Why It Exists / When It’s Used

Use a drain auger for:

- Clogged sinks, tubs, showers, and laundry drains

- Hair, grease, scale, or foreign objects stuck in traps

- Slow drains despite chemical “solutions”

- Root intrusion in sewer laterals (with larger models)

It’s the difference between surface cleanup and real plumbing.

## 3. Mastery Signals (What Pros Do Differently)

Pros:

✔ Feed cable slowly—never ram

✔ Rotate before retracting to avoid cable kinks

✔ Protect fixtures with towels + base pads

✔ Use trap removal to prevent cable damage

✔ Inspect with a camera when suspecting deeper line issues

Amateurs:

❌ Push cable aggressively = coil memory + pipe damage

❌ Skip gloves = shredded fingers

❌ Leave debris in trap = callback waiting to happen

❌ Use drill-powered snakes incorrectly = broken traps

TradeScout Culture Note:

“Pressure solves nothing. Patience solves drains.”

## 4. Types of Augers

| Type | Use Case | Notes |
| --- | --- | --- |
| Hand Auger | Small drains | Under sinks & tubs |
| Closet Auger | Toilets only | Won’t scratch porcelain |
| Drum Auger | Long runs | Professional-grade |
| Sewer Auger | Main lines | Root-capable, heavy gear |

A pro knows which one belongs where.

A scammer uses one tool for everything.

## 5. Safety Traps

⚠ Don’t snake through chrome traps — remove them first

⚠ Gloves + closed sleeves mandatory (moving cable hazard)

⚠ Hidden screws can catch cable and twist wrists

⚠ Never mix augers with drain chemicals — chemical burns explode

Homeowner rule:

If a contractor opens a drain with no PPE, don’t hire them.

## 6. Maintenance Checklist

- Wipe cable with oil after each use

- Drain moisture from drum

- Inspect kinks monthly

- Replace cables when rusted or frayed

A clean auger is a reliable auger. A rusty one is a lawsuit.

## 7. Recommendation Boosters

Tradespeople earn trust when they:

📌 Provide before & after flow test videos

📌 Show clog retrieval (hair, toys, etc.) — visual proof

📌 Leave a drain maintenance sheet (enzyme schedule)

📌 Photograph P-trap cleaning

The difference between “fixed” and “trusted” is documentation.