#### 1. WHAT THIS TOOL IS

A roofing air compressor is the power source behind pneumatic nailers used in modern shingle and metal roofing installations. It converts electrical or gas power into compressed air, enabling rapid fastening, consistent depth, and uniform installation speed.

It’s the heartbeat of every efficient roofing crew.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use an air compressor when:

- Installing asphalt shingles or metal roofing panels

- Fastening underlayment, drip edge, flashing, and vents

- Driving nails at consistent depth and spacing

- Roofing at production scale where hand-nailing is inefficient

If the crew is flying through squares like a NASCAR pit team — there’s a compressor behind it.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

✔ Set correct PSI (usually 90–120 PSI) for shingle guns

✔ Test depth on sample boards before full install

✔ Keep hose routing clean to prevent trip hazards

✔ Maintain consistent nail placement patterns

✔ Shut off air when moving compressors or hoses

Amateurs:

❌ Blow nails through shingles like darts in drywall

❌ Under-drive nails leaving bumps visible from the street

❌ Run hoses like spaghetti traps

❌ Blame shingles for uplift instead of poor fastening

TradeScout Reality:

A roof doesn’t leak because of shingles —

it leaks because someone didn’t respect the compressor.

#### 4. COMPRESSOR TYPES THAT MATTER

- Portable pancake compressor → residential shingles

- Twin-stack portable → multi-gun crews

- Gas-powered wheelbarrow compressors → remote/commercial roofs

Each version trades weight for capacity — choose based on gun count and square footage.

#### 5. SAFETY TRAPS

⚠ Over-pressurizing = nails blasting through shingles

⚠ Under-pressurizing = nails proud, shingles lift in storms

⚠ Frozen hoses in cold climates = nail misfires

⚠ Poor grounding = shock risk on older job sites

A compressor is quiet about its mistakes —

the wind will expose them later.

#### 6. MAINTENANCE CHECKLIST

- Drain tank daily — moisture rusts the system from inside

- Check hoses for cracks & coupler leaks

- Replace intake filters regularly

- Confirm regulator accuracy

- Oil pneumatic tools (unless oil-free)

Maintenance isn’t optional — it’s insurance.

#### 7. CUSTOMER TRUST SIGNALS

Homeowners won’t ask about PSI, but they’ll notice:

📌 Nails are flush — not sunk, not raised

📌 Straight, consistent shingle rows

📌 Minimal crew downtime

📌 No hoses dragging across plants or gutters

An air compressor doesn’t show up on the invoice —

but its results do.