#### 1. WHAT THIS TOOL IS

A roofing torch is a high-BTU propane flame tool used to heat, bond, and seal modified bitumen roofing membranes. It melts asphalt layers to form a waterproof, monolithic surface — no seams, no guesswork, no leaks.

It’s fire, pressure, and precision — all in one hand.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use a roofing torch for:

- Installing torch-down modified bitumen systems

- Repairing commercial flat roofs

- Flashing penetrations, drains, and parapet edges

- Re-sealing vulnerable areas exposed to pooling water

- Creating watertight seams without adhesives

If the roof is flat and built to endure sun, time, and storms, a torch probably touched it.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

✔ Maintain proper temperature — material glistens, not burns

✔ Keep torch moving to avoid blistering

✔ Roll membranes smoothly for uniform adhesion

✔ Understand drip edge, flashings, and overlaps

✔ Create welds that outlast the building beneath them

Amateurs:

❌ Overheat the membrane → scorched surface and leaks

❌ Trap air → blistering and roof failure

❌ Ignore drainage → ponding water becomes a lawsuit

❌ Melt flashing like marshmallows at a campfire

TradeScout Truth:

A torch doesn’t make a roofer a professional — restraint does.

#### 4. TORCH TYPES THAT MATTER

- Standard torch head → general membrane welding

- Wide-flame torch → large surface areas

- Detail/angle torch → corners, edges, penetrations

There’s no universal torch — match the flame to the mission.

#### 5. SAFETY TRAPS

⚠ Igniting insulation below decking = structural fire

⚠ Using near solvents or tar buckets = explosion risk

⚠ Failing to wet down surroundings in dry areas

⚠ Unattended flame = criminal negligence

Rule:

If your pulse isn’t fully present, don’t pick up the torch.

#### 6. MAINTENANCE CHECKLIST

- Check hoses for cracks and leaks

- Tighten regulator fittings

- Clean nozzle carbon buildup

- Purge line before ignition

- Store tank upright — always

A torch fails slowly… and then all at once.

#### 7. CUSTOMER TRUST SIGNALS

Homeowners (and commercial clients) will notice:

📌 No scorch marks on walls or siding

📌 Fire extinguisher on-site — visible and ready

📌 Crew works calmly, no torch panic

📌 Finished roof looks smooth, uniform, and thick

A torch job done well feels engineered, not guessed.