## SECTION 1 — WHAT FIXTURE INSTALL MEANS

Fixtures are what the customer sees—faucets, toilets, tubs, showers, sinks.

This stage turns plumbing from a concept into a working system.

Rough-in is the bones.

Fixtures are the handshake.

## SECTION 2 — STANDARD FIXTURE SET

- Toilet

- Lavatory sink

- Kitchen faucet

- Tub/shower valve

- Laundry hookups

- Water heater (electric/gas)

If water touches it, a plumber finishes it.

## SECTION 3 — TOOLS REQUIRED

- Adjustable wrench

- Supply line set

- Putty / silicone

- Wax ring / flange

- Plumber’s tape

- Caulk gun

## SECTION 4 — STANDARD FIXTURE WORKFLOW

### Example: Toilet Install

- Position flange

- Set wax ring

- Seat toilet (dead level)

- Connect supply

- Check for leaks

- Caulk base (front only so leaks reveal themselves)

### Example: Faucet Install

- Secure base plate

- Insert lines & valves

- Seal surfaces

- Flush sediment before final connection

## SECTION 5 — COMMON MISTAKES

- Over-tightened supply lines

- Caulked toilet base fully (hides leaks)

- Cross-threaded fittings

- Wrong valve height behind wall

Fixtures don’t leak because of bad parts—they leak because someone rushed.