TradeScout | Advanced Siding Techniques & Weather Management

Version: v1

### 1. PURPOSE

This guide explains the invisible part of siding — the part that prevents mold, rot, callbacks, lawsuits, and insurance claims.

Most siding looks good on day one.

Only correctly detailed siding looks good in year ten.

This file exists to make TradeScout users the latter.

### 2. THE PRIME DIRECTIVE

Water always wins unless you give it a way out.

Siding is not waterproof. It is a cladding — a shell that sheds bulk water while the drainage system handles the rest.

If you don’t control water movement, the wall assembly becomes a compost heap.

### 3. THE CONTROL LAYERS

Every exterior wall has four jobs:

| Layer | Job |
| --- | --- |
| Water control | Keep bulk water out |
| Air control | Stop airflow through walls |
| Thermal control | Keep conditioned air inside |
| Vapor control | Manage moisture movement |

Confuse these, and you inherit the homeowner’s future repairs.

### 4. WEATHER-RESISTIVE BARRIERS (WRBs)

These are the real protection — siding merely hides them.

Types include:

- House wrap (Tyvek, similar products)

- Integrated sheathing systems (ZIP, DensElement)

- Fluid-applied membranes

- Felt and asphalt papers (legacy but still common)

RULE: The WRB must be continuous, drained, and lapped correctly.

Reverse laps = rot + regret.

### 5. FLASHING IS NOT OPTIONAL

Flashing is water choreography.

Key locations:

- Window heads

- Sill pans

- Vertical joints

- Roof-to-wall intersections

- Deck attachments

- Pipe & vent penetrations

Flashing is a system, not a product.

Most installers buy it — few understand it.

### 6. RAINSCREENS

Rainscreens are the cheat code for durable siding.

A rainscreen:

- Creates an air gap behind siding

- Lets walls drain AND dry

- Extends siding lifespan

- Reduces paint failures

When a wall can dry, failure becomes optional.

### 7. EXPANSION AND MOVEMENT

Different materials move at different rates:

- Vinyl expands aggressively with heat

- Fiber cement moves minimally but cracks if forced

- Metal expands and contracts daily

- Wood swells with moisture, shrinks with dryness

If you don’t plan for movement, the house will move for you — violently.

### 8. PENETRATIONS

A wall is only as strong as the holes in it.

Each penetration needs:

- Back dam

- Flashing

- Siding termination

- Sealant where appropriate

Caulk is not waterproofing — it is a timer.

### 9. WHERE CONTRACTORS FAIL

The most common sins:

- Thinking siding is waterproof

- Relying on caulk instead of drainage

- Forgetting to flash electrical/meter penetrations

- Ignoring manufacturer clearances

- Cutting corners at transitions and terminations

Every siding failure is a lesson someone ignored.

### 10. WHY THESE TECHNIQUES MATTER TO TRADESCOUT

These details:

- Reduce callbacks

- Increase recommendation velocity

- Produce visible excellence

- Separate pros from installers

- Make homeowners evangelists

The siding contractor who understands why details matter becomes the highest-ranked professional in any TradeScout market.

Siding isn’t a trade — it’s a reputation system.