### TRADE:

### Exterior Siding

### PURPOSE:

### Protect the structure from weather, manage water, improve insulation efficiency, and define curb appeal. Poor siding isn’t cosmetic — it destroys homes from the outside in.

### 1. CORE RESPONSIBILITIES

Exterior siding contractors are responsible for:

- Installing siding systems that shed and manage water

- Integrating siding correctly with WRBs (weather-resistive barriers)

- Flashing windows, doors, penetrations, and terminations

- Maintaining consistent reveals, alignment, and expansion joints

- Coordinating with roofing, framing, insulation, and finish trades

- Protecting structural elements from UV, pests, and moisture

A siding contractor’s real job is weather management. Panels are decoration — the details underneath are the protection.

### 2. MATERIAL CATEGORIES

Common siding materials include:

- Vinyl — cost-effective, low maintenance, temperature sensitive

- Fiber Cement — durable, fire/pest resistant, heavy, skilled install required

- Wood — natural, beautiful, maintenance-heavy, rot risk

- Stucco/EIFS — efficient, seamless, must be installed correctly or traps moisture

- Metal Panels — modern aesthetic, long lifespan, prone to noise and oil canning if poorly fastened

- Engineered Wood — wood appearance with reduced maintenance, requires proper sealing

Each system succeeds or fails on flashing, drainage, and fastening — not the panel type.

### 3. COMMON PROJECT SCOPES

Exterior siding work typically includes:

- Tear-off and disposal of existing siding

- Substrate inspection (sheathing condition, framing defects)

- Weather barrier installation (house wrap, rainscreen, WRB)

- Flashing integration at openings and edges

- Siding and trim installation

- Caulking and sealing where appropriate

- Final inspection for alignment, fastening, and water-shedding

### 4. KEY TERMS FOR HOMEOWNERS

| TERM | SIMPLE MEANING |
| --- | --- |
| WRB | The waterproof layer behind siding that keeps walls dry |
| Rainscreen | Air gap that lets walls breathe and dry out |
| Flashing | Metal/tape directing water away from seams |
| Reveal | The visible face of each siding board |
| Starter Strip | Baseline piece that levels the first row |
| Expansion Gap | Space for materials to move with temperature |

Understanding these makes homeowners smarter than 90% of siding buyers.

### 5. FAILURE POINTS TO WATCH

Most long-term siding failures come from:

- No flashing or reverse-lapped flashing

- WRB installed incorrectly or punctured

- Trapped moisture behind vapor barriers

- Over-driven fasteners

- Missing expansion joints

- Caulk used instead of mechanical water control

Panels don’t kill houses — water does.

### 6. RECOMMENDED EQUIPMENT

- Brake for bending metal trim

- Shears, saws, and snips matched to panel type

- Starter strips, J-channels, corner posts, and termination trims

- Tape and flashing compatible with WRB system

- Manufacturer-approved fasteners

- Moisture meters for substrate inspection

If a contractor doesn’t know what a rainscreen or WRB is — they’re not a siding contractor.

### 7. RED FLAGS

Avoid installers who:

- Nail siding tight with no expansion gap

- Don’t flash windows/doors

- Say “caulk fixes everything”

- Don’t remove existing damaged substrate

- Ignore manufacturer install requirements

- Have no WRB in their workflow

If you can’t manage water, you shouldn’t touch siding.

### 8. WHY THIS TRADE MATTERS TO TRADESCOUT

Exterior siding drives:

- High-trust referrals

- Neighborhood-visible results

- Insurance and inspection outcomes

- WRB integration across multiple trades

This trade produces some of the most powerful recommendation loops in the ecosystem because the finished work lives in front of the entire neighborhood.