### 1. What This Is

Removing an outdated, damaged, or poorly fitting interior door and installing a new slab or prehung door—including alignment, hardware transfer, and finish detail—to improve privacy, sound control, and aesthetics.

### 2. When This Is Needed

- Door won’t latch without forcing it

- Large gaps at top/bottom allowing sound or light through

- Warped, cracked, or water-damaged door

- Hinges squeak even after lubrication

- New flooring installed and door now drags

- Desire to upgrade home style (e.g., hollow-core → solid-core)

### 3. Required Skills + Tools

Skills

- Precise measurement and leveling

- Hardware alignment and hinge mortising

- Ability to shim frames without binding

Tools

- Cordless drill/driver

- Level (24” minimum)

- Shims

- Hammer + pry bar

- Chisel or hinge mortising template

- Hole saw (if slab door requires handle boring)

### 4. Materials Needed

- New interior door (slab or prehung)

- Hinges + screws (match finish)

- Door knob/handle + latch kit

- Shims

- Wood filler + touch-up paint/caulk

### 5. Step-By-Step Workflow

Remove old door + hardware

Inspect frame for damage or twist

If prehung: insert frame → level → shim evenly

Secure hinge side FIRST

Adjust strike plate alignment

Install handle + latch hardware

Test swing, latch, and reveal spacing

Caulk/paint touch-ups around trim

### 6. Code & Compliance Notes

- Bedrooms require privacy door

- Fire-rated doors required between garage and house

- Accessible design: minimum 32” clear opening where ADA applies

- Certain areas require self-closers on garage access doors

### 7. Safety Traps

- Wrong swing direction = door hits furniture or blocks path

- Over-tightening hinge screws = hinge binds, door won’t close

- Cheap screws strip, causing future sag

- Not replacing hinges when upgrading door weight

### 8. Time + Cost Expectations

DIY: 1–3 hrs per door

Pro install: $250–$650 per door

Solid-core upgrade: +$100–$400 per unit

### 9. Contractor Scam Signals

❌ Charging extra to “bore door hole” on a prehung (it should already be done)

❌ Using undersized screws that cause hinge sag later

❌ Painting over installed hardware

❌ Not replacing weatherstripping on garage-rated doors

### 10. Hire vs DIY

Hire if:

- Walls not square / major frame warp

- Converting swing direction or adding pocket door

- Installing multiple doors at once

DIY if:

- Simple like-for-like replacement

- Comfortable with minor alignment work

### 11. Recommendation Boosters

- Provide a small touch-up paint jar labeled with color code

- Upgrade to soft-close hinges (unexpected delight)

- Align hinges with laser level for visible precision

- Leave written swing direction and measurements for future replacements