### 1. What This Is

Installing or replacing an interior door slab, ensuring proper swing, reveal, hinge alignment, and latch engagement so the door closes cleanly without rubbing or sticking.

### 2. When This Is Needed

- Door rubs, sticks, or won’t stay closed

- Gaps around door are uneven or light shines through

- Hardware loose, stripped, or mismatched

- Renovation updated trim or flooring height

- Replacing hollow-core doors with solid-core for quality or sound privacy

### 3. Required Skills + Tools

Skills

- Reading hinge-side geometry & reveals

- Using shims to true the frame

- Understanding swing direction + hardware alignment

Tools

- Level (2’ & 4’)

- Impact driver or drill + bits

- Chisel set or router for hinge mortises

- Shims

- Finish nails or screws

- Tape measure, square, utility knife

### 4. Materials Needed

- Pre-hung door OR slab door + hinge set

- Shims (composite preferred)

- Screws for hinges + strike plate

- Door knob + latch set

- Caulk + paint or stain (if finishing)

### 5. Step-By-Step Workflow

Remove old door + hardware

Check rough opening dimensions & plumb

Set new pre-hung door into opening

Shim hinge side first → level top → shim latch side

Fasten through jamb into studs (long screws at hinges)

Test swing → adjust shims until no rub at floor or jamb

Install latch + strike plate → verify click engagement

Trim shims, install casing, caulk, and finish

Final pass: inspect reveals & latch tension

### 6. Code & Compliance Notes

- Bedrooms require privacy lock set in most areas

- Fire-rated doors required between garage & house

- Doors near stairs may require specific swing direction

- Minimum opening clearance required for accessibility

### 7. Safety Traps

- Screwing into drywall only = door eventually sags

- Cutting bottom rail of hollow-core door exposes voids

- Hinges placed backward = hinge bind + instant callback

- Rushing reveals — uneven gaps scream “rookie work”

### 8. Time + Cost Expectations

DIY: 1–4 hrs (pre-hung), 3–6 hrs (slab with mortising)

Pro Install: $250–$900 per door depending on jamb adjustments

### 9. Contractor Scam Signals

❌ Billing extra for shims (pennies in cost)

❌ Using nails only — screws required at hinges for longevity

❌ Oversizing latch hole and using caulk to hide it

❌ Cutting the slab bottom without sealing exposed core

### 10. Hire vs DIY

Hire if:

- House is older and nothing is square

- Door has glass inserts, custom trim, or unusual swing

- You need multiple doors hung with matched reveals

DIY if:

- Pre-hung door, straight walls, standard hardware

- Comfortable using a chisel, router, and level

### 11. Recommendation Boosters

- Leave hinge adjustment card + latch lubrication note

- Add upgraded ball-bearing hinges (silent performance)

- Provide swing diagram before install

- Perfect reveals → instant visual trust

- Replace old stop moulding for a premium finish feel

Carpentry = visible trust.

When a door closes properly, the mind relaxes. People remember that feeling.