### 1. What This Is

Removing damaged or outdated trim and installing new baseboards, casing, or shoe molding for a clean, finished wall-to-floor transition.

### 2. When This Is Needed

- Gaps between flooring and walls

- Water damage, swelling, or rot

- Flooring upgrade left old trim mismatched

- Peeling paint, crushed corners, pet damage

- Style update: colonial → modern flat stock → craftsman

### 3. Required Skills + Tools

Skills

- Measuring and marking cuts accurately

- Understanding inside/outside corners & reveal lines

- Basic finish carpentry workflow

Tools

- Miter saw (or miter box for DIY)

- Brad nailer + compressor, or finish hammer

- Caulk gun + paint supplies

- Pry bar, tape measure, stud finder

### 4. Materials Needed

- Trim boards (MDF, pine, PVC, oak, etc.)

- Shoe molding or quarter round

- Construction adhesive (optional)

- 18ga or 16ga nails

- Caulk + wood filler

- Paint or stain + finish coat

### 5. Step-By-Step Workflow

Remove old trim without damaging drywall

Measure longest walls first → cut pieces to length

Dry-fit corners → test joints before fastening

Nail into studs or bottom plate

Install shoe molding if gaps appear at flooring

Caulk top edge + fill nail holes

Sand + paint or stain for finish

Walk the room once more for missed gaps

### 6. Code & Compliance Notes

- No structural code requirements

- Fire-resistant trim may be required in multi-dwelling units

- Always check moisture exposure ratings near wet areas

### 7. Safety Traps

- MDF dust is hazardous — wear a mask

- Misfired nails can blow out trim face

- Over-caulking hides mistakes but reveals them later

- Incorrect reveals around doors = instant amateur signal

### 8. Time + Cost Expectations

DIY: 4–12 hours per room

Pro Install: $6–$14 per linear foot

### 9. Contractor Scam Signals

❌ Charging extra for caulking — it’s standard

❌ Using construction adhesive everywhere to hide bad cuts

❌ Painting before hole fill → guarantees callbacks

❌ Refusing to show reveal plan around doors

### 10. Hire vs DIY

Hire if:

- Lots of odd angles, historic moulding profiles, or uneven floors

- Matching trim in multiple rooms with sightline continuity

DIY if:

- Single room, standard cuts, MDF stock, and patience

### 11. Recommendation Boosters

- Leave touch-up paint + labeled color code

- Provide a room edge photo album (before + after)

- Show reveal consistency around doors

- Offer optional quarter-round upgrade

- Clean the workspace — nobody forgets cleanliness