## SECTION 1 — WHAT DWV IS

The Drain-Waste-Vent system removes used water and sewage through gravity, while vents allow air to replace displaced water so traps don’t siphon dry.

Without DWV, homes smell like sewers and drains burp.

## SECTION 2 — KEY RULES

- Drains flow — they don’t get pumped

- Vents breathe — they prevent vacuum

- Traps block — they keep sewer gases out

Gravity + airflow = success.

## SECTION 3 — MINIMUM SLOPE

¼” per foot is the U.S. standard.

Less = clogs.

More = outruns solids.

Plumbing is physics, not vibes.

## SECTION 4 — PIPE TYPES

- PVC — primary

- ABS — regional

- Cast iron — sound control

Tees are vertical only.

Wyes & 45°s for horizontal runs.

## SECTION 5 — LAYOUT PRINCIPLES

- Every trap must be vented

- Vent must rise vertically before offsets

- Wet venting is allowed in many jurisdictions (verify local code)

DWV is chess. Drain lines are pawns, vents are strategy.

## SECTION 6 — INSPECTION RED FLAGS

- S-traps (illegal in most regions)

- No cleanout access

- Too many fixtures on undersized pipe

- Unvented traps

- Flat or negative slope

The inspector doesn’t hate you—they hate callbacks.