## SECTION 1 — THE FIRST RULE

# You can stack insulation for days, but if air moves freely, your R-value is make-believe.

# Heat rides air.

# Moisture rides air.

# Smells ride air.

# Stop the air, stop the problems.

## SECTION 2 — WHERE BUILDINGS LEAK

# Electrical boxes

# Top plates

# Can lights

# Attic hatches

# Plumbing penetrations

# Rim joists

# Window & door interfaces

# Most energy loss is not in walls — it’s in holes someone forgot to seal.

## SECTION 3 — AIR SEALING MATERIALS

# Caulk — cracks and seams

# Spray foam — penetrations

# Weatherstripping — moving parts

# Tapes & membranes — continuous barriers

# Air control is done with tools cheap enough to embarrass the status quo.

## SECTION 4 — BEST PRACTICES

# Seal first, insulate second

# Respect drying paths

# Use backer rod for big joints

# Verify with blower-door logic when possible

# Good builders measure leaks. Great builders prevent them.