### 1. WHAT THIS IS

Installing Luxury Vinyl Plank (LVP) flooring, including subfloor prep, layout planning, plank cutting, and expansion spacing to produce a waterproof, durable finished floor.

### 2. WHEN THIS IS NEEDED

- Existing flooring is damaged, stained, or outdated

- Homeowner wants waterproof, pet-proof, or kid-proof flooring

- Transition from carpet to hard surface

- Basement or slab flooring upgrade where moisture is a concern

### 3. REQUIRED SKILLS + TOOLS

Skills

- Accurate measuring & layout

- Basic cutting (utility knife, oscillating tool, miter saw)

- Understanding expansion gaps & transitions

Tools

- Tape measure

- Utility knife

- Rubber mallet

- Pull bar & tapping block

- Spacers

- Square & pencil

- Optional: miter saw, jamb saw, oscillating tool

### 4. MATERIALS NEEDED

- LVP flooring planks

- Underlayment (if product requires)

- Moisture barrier (for slab installs)

- Quarter-round or base molding

- Transition strips

- Shims/spacers

### 5. STEP-BY-STEP WORKFLOW

Acclimate flooring 24–48hrs

Check subfloor flatness → Fill dips, sand high spots

Remove baseboards (optional)

Install moisture barrier if on slab

Plan layout → Avoid small slivers on last row

Start along longest exterior wall

Use spacers for 1/4” expansion gap

Click-lock planks: tap lightly, don’t force

Stagger seams (6”+, avoid alignment repeats)

Cut planks at doors & transitions

Install transitions & reinstall trim

Vacuum + clean surface

### 6. CODE & COMPLIANCE NOTES

- Maintain manufacturer-required expansion gap

- Do not trap flooring under cabinets (floating floor rules)

- Moisture barrier mandatory over concrete

- Follow stair nosing requirements if used on treads

### 7. SAFETY TRAPS

⚠️ Cutting indoors creates microplastic dust — use mask

⚠️ Failing to acclimate = buckling and warranty denial

⚠️ No expansion gap = flooring peaks or separates

⚠️ Installing under fixed objects locks floor in place

### 8. TIME + COST EXPECTATIONS

DIY: 1–3 days (average room)

Pro install: $3–$8/sq ft labor + materials

### 9. CONTRACTOR SCAM SIGNALS

❌ “Underlayment always required” — depends on plank system

❌ Selling transitions you don’t need

❌ No moisture testing on concrete slabs

### 10. HIRE VS DIY

Hire a pro if

- Subfloor needs leveling

- Flooring crosses multiple rooms without breaks

- Floating floor interacts with built-ins

DIY if

- Single room, flat subfloor, standard trim

### 11. RECOMMENDATION BOOSTERS

- Leave a plank box with serial info & care sheet

- Offer furniture sliders

- Provide cleaning instructions + 12-mo check-in