### 1. WHAT THIS IS

Installing ceramic or porcelain floor tile over a properly prepared substrate, creating a waterproof, durable, and long-lasting surface for bathrooms, kitchens, laundry rooms, and entry areas.

### 2. WHEN THIS IS NEEDED

- Existing floor is damaged, outdated, or water-sensitive

- Homeowner wants a flooring surface that resists spills, pets, and heavy wear

- Moisture-prone area (bathroom, laundry, mudroom) needs a non-absorbent surface

- Cracked tiles or loose grout in existing space

- Subfloor is sagging, soft, or moves when stepped on (requires remediation before install)

### 3. REQUIRED SKILLS + TOOLS

Skills

- Leveling substrates and identifying flex

- Basic cutting using tile saw or snap cutter

- Understanding thinset mortar behavior and grout spacing

Tools

- Wet tile saw or snap cutter

- Notched trowel

- Tile spacers

- Mixing bucket + paddle

- Level + straightedge

- Grout float + sponge

- Knee pads (trust us)

### 4. MATERIALS NEEDED

- Tile (porcelain preferred for durability)

- Thinset mortar (DO NOT use mastic on floors)

- Cement board or uncoupling membrane (DITRA, etc.)

- Screws + backer board tape

- Grout (sanded for ≥1/8” joints, unsanded for smaller)

- Silicone caulk for perimeter + wet zones

- Tile spacers (1/8” typical)

### 5. STEP-BY-STEP WORKFLOW

Remove old flooring + clean subfloor

Check for movement — fix weak subfloor

Install cement board with correct screws

Tape seams with thinset

Dry-lay tile to map layout

Mix mortar to peanut-butter texture

Trowel thinset in straight lines (NOT swirls)

Press tile firmly and use spacers

Cut tiles as needed along edges

Allow cure (24 hrs)

Apply grout and pack joints fully

Wipe haze after grout sets

Caulk all corners and perimeter joints

Install transitions + reinstall base trim

### 6. CODE & COMPLIANCE NOTES

- Cement board must be supported by thinset — screws alone are NOT compliant

- Movement joints required every 20–25 feet in large areas

- Wet areas require waterproofing membrane behind or under tile

- Grout is not waterproof; perimeter silicone required

### 7. SAFETY TRAPS

⚠️ Installing tile over OSB without backer board → cracking guaranteed

⚠️ Using premixed “floor adhesive” → not real thinset, fails under load

⚠️ Swirl troweling → creates air pockets, cracked tile later

⚠️ No expansion gap at walls → tile tenting

### 8. TIME + COST EXPECTATIONS

DIY: 2–4 days (prep + install + cure)

Pro Install: $10–$35 per sq ft depending on tile complexity

Tools rental (saw): $40–$80/day

### 9. CONTRACTOR SCAM SIGNALS

❌ Tiling directly over plywood without membrane

❌ Using mastic instead of mortar for floors

❌ Grouting corners instead of caulking

❌ No movement joints in large areas

### 10. HIRE VS DIY

Hire if:

- New tile intersects multiple rooms

- Patterned tile requiring alignment

- Subfloor flex detected

- Wet-zone waterproofing needed

DIY if:

- Simple rectangular room

- Straight lay pattern

- Basic cuts and transitions

### 11. RECOMMENDATION BOOSTERS

- Laser-straight lines → visible craftsmanship

- Leave leftover grout + color code

- Seal grout after cure and provide care guide

- Offer annual re-seal service for bathrooms