### 1. What This Is

Upgrading a standard furnace/air handler filter to a higher-performance filtration solution—such as a HEPA cabinet, MERV 11–16 media filter, or electronic air cleaner—to improve indoor air quality, protect HVAC equipment, and reduce dust, allergens, and particulates.

### 2. When This Is Needed

- Allergy or asthma symptoms in the home

- Excessive dust buildup on furniture or vents

- Current filter rated below MERV 8

- Visible dirt on blower fan or evaporator coil

- Home has pets, smokers, woodworking hobby, or high pollen exposure

- New HVAC system installed without upgraded filtration

### 3. Required Skills + Tools

Skills

- Understanding airflow direction

- Identifying filter cabinet size and duct compatibility

- Basic electrical disconnect (electronic cleaners only)

Tools

- Nut driver / screwdriver

- Tape measure

- Utility knife (for sealing gaps)

- Aluminum foil tape (NOT duct tape)

### 4. Materials Needed

- New upgraded filter or filtration cabinet

 (MERV 11–16, HEPA, electrostatic, or electronic air cleaner—choose based on home needs)

- Replacement filters (6–12 month supply recommended)

- Foil HVAC tape for sealing air leaks

### 5. Step-By-Step Workflow

Turn off HVAC power

Remove old filter → check airflow arrow direction

Measure filter slot and note system tonnage

Choose filter type based on household needs

Install new cabinet (if upgrading beyond disposable filter)

Seal duct gaps around cabinet to prevent bypass

Insert upgraded filter and confirm proper fit

Restore HVAC power and run system

Verify no whistling or airflow restrictions

### 6. Code & Compliance Notes

- Minimum filtration requirements vary by state and building type

- Filters must not reduce airflow below manufacturer spec (risk: frozen coils)

- Electronic cleaners may require dedicated power

- Some jurisdictions mandate MERV 13+ for multi-family / commercial buildings

### 7. Safety Traps

- Oversizing filtration → chokes airflow = equipment failure

- Bypass gaps → “filter upgrade” does nothing if air flows around it

- Cheap pleated filters → restrict air like a HEPA without benefits

- Ignoring coil cleanliness → filters can’t solve existing contamination

### 8. Time + Cost Expectations

DIY: 15–60 minutes

Pro Install: $350–$2,600 depending on cabinet type + electrical work

Replacement filters: $25–$300 each depending on HEPA vs media

### 9. Contractor Scam Signals

❌ Selling “lifetime filters” that are impossible to clean

❌ Adding UV lights without explaining why (not filtration; microbial only)

❌ Calling MERV 20 a thing — it doesn’t exist

❌ Installing oversized cabinet that reduces airflow

### 10. Hire vs DIY

Hire if:

- Installing HEPA bypass system

- Electronic filter requiring wiring

- Duct modifications needed for cabinet fit

DIY if:

- Swapping standard filter size and upgrading MERV level

- Cabinet already exists and you’re changing filter media only

### 11. Recommendation Boosters

- Add system airflow reading before & after upgrade

- Offer subscription filter deliveries every 6 months

- Provide filter change reminders (QR code or app-based)

- Leave static pressure readings to prove no airflow loss

Good filtration is not a luxury — it’s HVAC armor.

If the lungs of the house can’t breathe, nothing else matters.