### 1. What This Is

Replacing an outdated thermostat with a modern programmable or smart model to improve comfort control, reduce energy costs, and enable automation features like remote access and learning schedules.

### 2. When This Is Needed

- Temperature swings or uneven comfort between rooms

- Old mercury thermostat (pre-2000 technology)

- Rising energy bills without usage changes

- System runs constantly or short cycles

- Homeowner wants Wi-Fi control, zoning, or voice integration

### 3. Required Skills + Tools

Skills

- Identifying HVAC control wires

- Understanding system type (heat pump vs conventional)

- Registering smart device + Wi-Fi pairing

Tools

- Screwdriver set

- Wire labels

- Drill (if new mounting holes required)

- Smartphone for app setup

### 4. Materials Needed

- New thermostat (smart or programmable)

- Backplate + mounting screws

- Wire labels (often included)

- Trim kit (optional for old paint lines)

### 5. Step-By-Step Workflow

Turn off HVAC power at breaker

Remove old thermostat → photograph wiring

Label wires by terminal (R, C, G, W, Y, etc.)

Attach new backplate → ensure level (smart thermostats are picky)

Connect wires to corresponding terminals

Install thermostat → power system on

Complete app onboarding + Wi-Fi pairing

Run heating/cooling test cycle

Configure schedules, away mode, and temperature limits

### 6. Code & Compliance Notes

- Smart thermostats must not override emergency heat mode

- Heat pump installs require special wiring for O/B terminals

- Some areas require minimum temperature settings for rentals

- System must maintain cooling lockout protections for heat pumps

### 7. Safety Traps

- NO COMMON WIRE = device won’t power properly

- Incorrect heat pump wiring = reversed hot/cold output

- Mounting crooked → thermostat tilt affects motion sensors

- Leaving batteries in obsolete wall cavity (fire risk)

### 8. Time + Cost Expectations

DIY: 30–90 minutes

Pro Install: $180–$600 depending on system type and wiring complexity

### 9. Contractor Scam Signals

❌ Adding a “mandatory C-wire module” when a common wire exists

❌ Selling proprietary thermostats requiring paid subscriptions

❌ Installing wrong mode for heat pump vs furnace

❌ Charging extra for app setup — takes 5 minutes

### 10. Hire vs DIY

Hire if:

- No C-wire present and walls need fishing

- Zoning system or heat pump with dual-fuel logic

- HVAC control board modifications required

DIY if:

- Replacing like-for-like thermostat with labeled wiring

- Confident using app setup + breaker shutoff

### 11. Recommendation Boosters

- Provide temperature schedule presets (Home / Away / Sleep)

- Enable usage reports for cost savings accountability

- Add humidity control if compatible

- Label wiring inside panel for the next homeowner

A thermostat is the brain of the house.

Upgrade it, and everything downstream behaves better.