### 1. WHAT THIS IS

Repairing cracks in concrete driveways, sidewalks, or walkways to prevent water penetration, trip hazards, and further structural deterioration.

### 2. WHEN THIS IS NEEDED

- Visible cracking or surface separation

- Trip hazards forming along crack edges

- Weeds growing through concrete gaps

- Water pooling in sections after rain

- Home inspection noted settlement issues

### 3. REQUIRED SKILLS + TOOLS

Skills

- Crack evaluation (static vs. moving)

- Bonding agent application

- Concrete finishing

Tools

- Angle grinder or chisel

- Caulk gun

- Wire brush + broom

- Pressure washer

- Trowel

### 4. MATERIALS NEEDED

- Concrete crack filler or polyurethane sealant

- Bonding adhesive

- Self-leveling compound (for dips)

- Expansion joint material (if missing)

### 5. STEP-BY-STEP WORKFLOW

Clean crack + remove loose debris

Widen crack to V-groove (improves bond)

Apply bonding agent

Fill crack with sealant or repair mix

Tool surface flush

Allow curing per manufacturer specs

Apply sealer if desired

Regrade soil along edges if pooling

### 6. CODE & COMPLIANCE NOTES

- Trip hazard thresholds vary (typically >1/4”)

- ADA walkway slope guidelines apply in some zones

- Concrete sealers cannot enter storm drains

### 7. SAFETY TRAPS

⚠ Surface patch without bonding = peels in 6 months

⚠ Filling movement joints solid = cracks return

⚠ Using interior caulk outdoors → UV failure

⚠ Acid wash runoff into drains = fines in many municipalities

### 8. TIME + COST EXPECTATIONS

DIY: 2–4 hrs

Pro repair: $250–$2,800

Full concrete replacement: $4,500–$22,000

### 9. CONTRACTOR SCAM SIGNALS

❌ Painting concrete to “hide cracks”

❌ No joint control cuts in replaced sections

❌ “Lifetime patch” claims without moisture test

❌ No talk of drainage or slope issues

### 10. HIRE VS DIY

Hire if:

- Cracks expand or change seasonally

- Multiple slabs sinking or heaving

- Large trip hazards present

DIY if:

- Hairline and non-moving cracks only

### 11. RECOMMENDATION BOOSTERS

- Provide slope/level check

- Seal driveway after repair

- Show before/after crack map

- Offer annual inspection subscription