### 1. WHAT THIS IS

Full repaint of the home’s exterior surfaces to protect against UV exposure, rot, moisture, and fading while enhancing curb appeal and property value.

### 2. WHEN THIS IS NEEDED

- Chalky residue when rubbing siding

- Color fading or peeling paint

- Bare wood exposure

- HOA compliance issues

- Preparing to sell home

### 3. REQUIRED SKILLS + TOOLS

Skills

- Proper prep on wood, stucco, fiber cement

- Masking windows + fixtures

- Understanding weather windows

Tools

- Pressure washer

- Exterior sprayer (optional)

- Brushes, rollers, ladders

- Scraper + sanding tools

### 4. MATERIALS NEEDED

- Exterior paint (latex recommended)

- Primer for raw areas

- Caulk for expansion joints

- Plastic + tape for masking

- Wood filler for damaged trim

### 5. STEP-BY-STEP WORKFLOW

Wash siding + scrape loose paint

Sand rough edges

Prime bare wood + patched spots

Caulk joints + gaps

Mask windows + fixtures

Apply paint (spray + backroll ideal)

Check coverage + trim crisp lines

Remove masking + clean site

### 6. CODE & COMPLIANCE NOTES

- Lead-safe procedures for pre-1978 homes (EPA RRP rule)

- HOA color approval required in many neighborhoods

- Ladder placement rules for public walkways

### 7. SAFETY TRAPS

⚠ Spraying in wind → overspray on cars/neighbors

⚠ Ignoring rotten trim before painting

⚠ One-coat exterior claims → lie

⚠ Skipping primer on raw wood

### 8. TIME + COST EXPECTATIONS

DIY: 3–7 days

Pro repaint: $3,500–$18,000 depending on size

Paint lifespan: 5–12 years depending on climate

### 9. CONTRACTOR SCAM SIGNALS

❌ “Free pressure washing” that damages siding

❌ No primer with color-change job

❌ Watered-down paint

❌ No written warranty

### 10. HIRE VS DIY

Hire if:

- Two-story home

- Extensive scraping

- Sprayer required

DIY if:

- Small rambler + easy access

### 11. RECOMMENDATION BOOSTERS

- Provide before/after moisture meter readings on wood

- Use high-build primer on old trim

- Offer annual touch-up for sun-exposed sides