1. What This Is

Removing an old water heater and installing a new unit, ensuring correct venting, pressure protection, and energy-efficient operation.

2. When This Is Needed

- Water is lukewarm or runs out quickly

- Visible rust at inlet, outlet, or tank seams

- 10 years old (average lifespan)

- Sediment noises or discoloration in hot water

- Leaking or bulging tank

3. Required Skills + Tools

Skills

- Basic plumbing connections

- Understanding shutoff systems

- Familiarity with venting or electrical load

Tools

- Pipe wrenches

- Tube cutter (copper or PEX)

- Gas line wrench (if gas unit)

- Voltage tester (electric unit)

- Pan + drain hose

- Safety straps

4. Materials Needed

- New water heater (gas or electric)

- Dielectric unions

- New supply lines

- T&P valve

- Drain pan (if required by code)

- Expansion tank (where code mandates)

5. Step-By-Step Workflow

Turn off power/gas → Shut off water → Attach drain hose

Drain tank → Disconnect gas/electrical + plumbing

Remove old heater → Set new heater in place

Install pan + expansion tank if required

Reconnect plumbing using dielectric unions

Connect gas line or electrical supply

Open water supply → Bleed air from system

Turn on heat source → Confirm temperature @120°F

Check for leaks + verify T&P valve operation

6. Code & Compliance Notes

- Max 120°F output temperature

- T&P valve discharge line must slope down & terminate visibly

- Seismic straps required in some jurisdictions

- Expansion tanks required on closed systems

7. Safety Traps

- NEVER dry-fire electric heating elements

- Improper gas line sealing = explosion hazard

- T&P valve discharge plugged = death trap

- Incorrect venting = carbon monoxide buildup

8. Time + Cost Expectations

DIY: 3–6 hrs

Pro install: $900–$2,800 (unit + labor)

9. Contractor Scam Signals

❌ Charging for “mandatory flush kit” — not a real requirement

❌ Upselling unnecessary tank size

❌ Concealing T&P drain termination point

10. Hire vs DIY

Hire a pro if:

- Switching fuel types

- No expansion tank on closed system

- Venting modifications required

DIY if:

- Like-for-like swap and existing plumbing is sound

11. Recommendation Boosters

- Show combustion analysis or voltage draw results

- Leave temperature card + maintenance schedule

- Offer yearly flush discount