### 1. WHAT THIS IS

Replacing damaged, missing, or worn asphalt shingles to restore roof integrity, prevent leaks, and extend the lifespan of the roofing system.

### 2. WHEN THIS IS NEEDED

- Shingles curling, cracking, or shedding granules

- Visible bald spots or missing shingles after a storm

- Water stains in attic or ceiling

- Roof older than 18–25 years

- Dark streaks or moss growth trapping moisture

### 3. REQUIRED SKILLS + TOOLS

Skills

- Safe ladder usage + roof navigation

- Basic fastening + flashing awareness

- Reading shingle alignment lines

Tools

- Roofing hammer / nail gun

- Flat bar (for shingle removal)

- Roofing nails

- Utility knife + hook blades

- Ladder + fall protection

### 4. MATERIALS NEEDED

- Matching shingles (brand, color, style)

- Roofing nails (galvanized)

- Roof cement / sealant

- Underlayment patch (if rot found)

- Flashing patches (optional)

### 5. STEP-BY-STEP WORKFLOW

Identify damaged shingles

Lift shingles above → remove nails

Slide out damaged shingle

Insert new shingle under existing layer

Align with guide lines

Fasten with 4–6 nails per manufacturer spec

Seal exposed tabs with roof cement

Check for loose adjacent shingles

Clean debris + final sweep

### 6. CODE & COMPLIANCE NOTES

- Max roof layers: typically 2; third layer illegal in most states

- Ice & water shield required in snow states

- Fasteners must penetrate decking by 3/4”

- No staples — nails only

### 7. SAFETY TRAPS

⚠ Wrong nail placement → blow-offs in high wind

⚠ Not sealing tabs → uplift failure

⚠ Working wet roof → slip & death risk

⚠ Mismatch shingles → buyer disputes

### 8. TIME + COST EXPECTATIONS

DIY patch: 1–3 hrs

Pro repair: $150–$650

Full replacement: $8,000–$28,000 depending on size + region

### 9. CONTRACTOR SCAM SIGNALS

❌ Claims “storm damage” on non-damaged roofs

❌ Missing manufacturer paperwork

❌ Install in cold temps without seal activation

❌ Reusing flashing without inspection

### 10. HIRE VS DIY

Hire if roof pitch is steep, multiple layers exist, or replacing valleys/rakes.

DIY if replacing isolated shingles on a walkable roof under 6/12 pitch.

### 11. RECOMMENDATION BOOSTERS

- Leave sealed photo log of replaced sections

- Provide ventilation check report

- Color-match certification from supplier

- Offer annual roof tune-up program