### TS_TOOL_CARPENTRY_CIRCULAR_SAW_v1

#### 1. WHAT THIS TOOL IS

A circular saw is a handheld power saw that uses a rotating blade to make straight cuts in wood, framing materials, subfloors, sheet goods, and trim. It is the core cutting tool in modern carpentry and jobsite construction.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use a circular saw when:

- Framing walls, floors, or roofs

- Cutting plywood, OSB, LVL beams, and lumber

- Trimming boards to length

- Making straight, repeatable cuts fast

If lumber is being shaped, shortened, or divided, the circular saw is involved.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

- Set blade depth to just below material thickness for safety and cleaner cuts

- Support workpiece fully to prevent blade binding

- Use a guide or track for long precision cuts

- Start blade outside material, allow it to hit full speed, then enter the cut

- Keep both hands engaged unless saw features safety support

Amateurs:

- Cut with full blade depth exposed (kickback hazard)

- Push too hard rather than letting blade do the work

- Don’t secure material, causing board vibration

- Use wrong blade type for wood, metal, or composites

TradeScout Culture Rule:

“Every cut is a decision. Every decision leaves a line you can never uncut.”

#### 4. COMMON BLADE TYPES

Rip blades for cutting with the grain

Crosscut blades for clean finish cuts

Combination blades for general work

Carbide tips last longer and resist nails and hidden debris

A good contractor knows which blade belongs in the saw before the saw touches wood.

#### 5. SAFETY TRAPS

Kickback from pinched blade

Loose clothing near blade path

Cutting unsupported material

Not wearing eye and hearing protection

Blade slowing down during cut equals danger

If a contractor cuts wood near a ladder, cord, or hand placement without planning the path, pause the hire.

#### 6. MAINTENANCE CHECKLIST

Clean sawdust from motor vents weekly

Replace dull blades immediately

Oil the base plate for glide

Check guard return action before each job

Ensure cord or battery is damage-free

A dull blade is more dangerous than a sharp one because friction creates force and force creates failure.

#### 7. CUSTOMER TRUST SIGNALS

Pros build instant confidence when they:

- Use a guide track or straight edge for sheet cuts

- Measure twice, mark once, cut once

- Leave clean edges without splintering

- Vacuum debris rather than leaving sawdust piles

Clean work equals clear competence. Sawdust management tells the real story.