#### . WHAT THIS TOOL IS

A roofing nail gun is a pneumatic or battery-powered fastening tool designed to drive roofing nails rapidly into shingles, underlayment, and roof decking. It replaces hand-nailing with controlled, repeatable impact — dramatically increasing speed and consistency.

It is the heartbeat of modern roofing production.

#### 2. WHY IT EXISTS / WHEN IT’S USED

Use a roofing nail gun when:

- Installing asphalt shingles, underlayment, felt, or drip edge

- Fastening ridge caps and starter rows

- Securing roof components where speed and uniform penetration matter

- You need hundreds to thousands of nails installed in a compressed timeframe

If you’re hand-nailing, you’re roofing in the past.

#### 3. WHAT PROS DO DIFFERENTLY

Professionals:

✔ Dial in PSI to avoid over-driving

✔ Keep the gun perpendicular for proper nail seating

✔ Adjust depth per shingle + substrate type

✔ Switch nail coils by size and weather requirements

✔ Maintain rhythm — gun never stops moving

Amateurs:

❌ Overdrive nails through shingles

❌ Angle the gun and cut through fiberglass mats

❌ Leave nails exposed to UV and wind tear-off

❌ Think more nails = better roof (it doesn’t)

TradeScout Commandment:

Roofing is not fastening — it’s sealing. The nail enforces the seal.

#### 4. NAIL TYPES THAT MATTER

- 1¼” – 1¾” coil roofing nails (galvanized)

- Ring shank for better pull resistance

- Hot-dipped galvanized for coastal zones

The wrong nail voids warranties. A pro knows this; a hack ignores it.

#### 5. SAFETY TRAPS

⚠ Gun can double-fire if the trigger rebounds

⚠ Nails ricochet if decking is soft or rotted

⚠ Running hoses across roof edges = fall hazard

⚠ Battery guns can misfire if debris blocks contact tip

The most dangerous tool on a roof is the one you don’t respect.

#### 6. MAINTENANCE CHECKLIST

- Oil pneumatic guns daily (unless oil-free)

- Drain compressor tank moisture

- Clean contact safety tip

- Inspect feed coil and driver blade

- Check hose fittings for leaks

A roofing crew with a broken nail gun becomes a tear-off crew with no timeline.

#### 7. CUSTOMER TRUST SIGNALS

Homeowners notice:

📌 Straight shingle lines

📌 Consistent nailing patterns

📌 No raised or exposed nail heads

📌 No visible blow-throughs

📌 Steady, confident workflow

A good roof is quiet confidence — the sound of precision, not panic.

This tool is how pros prove they’re pros.