## 1. PURPOSE OF LADDER SAFETY

Falls are the #1 cause of contractor injury.

Most ladder accidents come from rushing, bad angles, or poor footing.

Ladder safety is non-negotiable.

If the ladder isn’t right, the job doesn’t start.

⸻

## 2. LADDER SELECTION RULES

✔ Use the right ladder height — never stand on the top step

✔ Choose fiberglass near electrical work

✔ Choose extension ladders for roof access

✔ Choose step ladders for indoor work

✔ Confirm weight rating matches the worker + tools

TradeScout Standard:

“The wrong ladder is a hazard before you climb it.”

⸻

## 3. LADDER SETUP RULES

Always set a ladder on:

✔ Level, solid ground

✔ Non-slip surfaces

✔ Clean areas free of debris, hoses, cords

Extension ladder angle rule:

✔ Base 1 foot out for every 4 feet of height (4:1 rule)

Tie-off rule:

✔ Secure the ladder when working at height or near traffic

Top access rule:

✔ Ladder must extend at least 3 feet above the landing point

If the ladder rocks, you don’t climb.

⸻

## 4. CLIMBING RULES

✔ Maintain 3 points of contact at all times

✔ Face the ladder when climbing

✔ Keep tools on a belt or hoisted — not in hands

✔ Move slowly and deliberately

✔ Never overreach — climb down and reposition

Overreaching is responsible for most ladder falls.

⸻

## 5. WORKING FROM A LADDER

When working:

✔ Keep your belt buckle between the ladder rails

✔ Don’t lean your body sideways

✔ Use stabilizers when needed

✔ Don’t carry heavy loads up the ladder

✔ Use platform ladders for extended work

If you can’t maintain balance while working, change the ladder.

⸻

## 6. WEATHER & ENVIRONMENT HAZARDS

Avoid ladder work when:

⚠ Ground is wet or icy

⚠ Wind exceeds safe working levels

⚠ Rain reduces tread grip

⚠ Electric lines are nearby

Weather changes ladder safety instantly.

⸻

## 7. INSPECTION CHECKLIST

Inspect ladders:

✔ Before every shift

✔ After transport

✔ After impacts or drops

Look for:

• Bent rails

• Cracked rungs

• Loose spreaders

• Slipping feet

• Missing hardware

• Damaged rope (extension ladders)

A damaged ladder is removed from service immediately.

⸻

## 8. STORAGE RULES

✔ Store ladders horizontally on racks

✔ Keep indoors when possible

✔ Protect fiberglass from sun damage

✔ Keep steps clean and dry

A ladder that lasts is a ladder that stays safe.

⸻

## 9. LADDER ALTERNATIVES

If a ladder is unsafe for the task, use:

• Scaffolding

• Work platforms

• Man lifts

• Step stools for short tasks

Choosing the right elevation tool is part of safety.

⸻

## 10. COMMON LADDER MISTAKES TO AVOID

❌ Standing on the top cap

❌ Using makeshift supports

❌ Climbing with hands full

❌ Placing ladder in front of doorways

❌ Leaning a step ladder like an extension ladder

❌ Skipping inspection

Small mistakes create major injuries.

⸻

## 11. CUSTOMER TRUST SIGNALS

Homeowners trust pros who:

📌 Set the ladder carefully and slowly

📌 Use stable footing and protective pads

📌 Never rush climbs or descents

📌 Move the ladder instead of overreaching

📌 Work with confidence and control

Safe climbing reassures homeowners immediately.